Release Notes PN 93000501

Digi Connect ME

82000856_C EOS

November 12,  2003

INTRODUCTION

This is a production release of firmware for the Digi Connect ME.

The Digi Connect ME embedded device server web and network enables

products easily without the complexities of extensive hardware and

software integration. The Digi Connect ME provides powerful

"plug-and-play", customizable and future-safe features, and

performance in one of the smallest solutions available.

SUPPORTED PRODUCTS

Digi Connect ME

ENHANCEMENTS

The ability to force the ethernet speed to a specific value

(rather than depending on auto-negotiation) has been added.

Auto-IP has been added as an IP address selection method.

See the section "IP ASSIGNMENT NOTES" below for more details.

Pseudo-modem capability has been added as a configurable

option for the serial port.

There are now two choices to use for web-enabled configuration;

the Java Configuration Applet (previously shipped with the

unit) and a new embedded web user interface.  The embedded web

user interface allows the user to configure the unit without

the need to download a Java Run-time Environment.  The Java

Configuration Applet is better suited for easy customization.

If you simply intend to access and configure the unit from a

web browser, you will want to use the embedded web user

interface.  If you want to create a custom UI, tailored to a

specific look and feel, use the Java Configuration Applet.

See the section "ENABLING THE EMBEDDED WEB USER INTERFACE" for

instructions on how to enable the embedded web user interface

as the default.

If you choose to use the Java Configuration Applet you must

upgrade the applet on your Digi Connect ME to ensure

compatibility between revisions.  See the section

"UPGRADING THE CONFIGURATION APPLET" for instructions.

Added a capability for embedded hosts to soft reset the Digi

module as well as reset the Digi module configuration back to

its factory default state via hardware signals.  See the

section "RESETTING THE UNIT" for details.

Added the "RTS Toggle" feature to the serial port configuration

(ability to control the "RTS" signal based on data transmission).

The web server is now configurable via the command line interface,

in that it can be enabled or disabled, as well as moved to a

different TCP port number.  Changing the port number or

disabling the web server will, as a side effect, cause certain

applications and applets to stop operating as they expect the

HTTP service to be at port 80.

The Digi RealPort service in the firmware has been extended to

fully support all Digi supported operating systems, not just

the Windows family of operating systems.

BUG FIXES

Two conditions were identified in which the unit would reboot

itself during early initialization as a result of internal

software conflicts.  These conditions have been eliminated.

The "Revert to Factory Defaults" feature exposed by the various

user interfaces have been consolidated so that their effect is

consistent.

There were cases in which the serial core would incorrectly

interpret signals as modem control signals even though the

user has identified them as GPIO inputs or outputs.  These

conditions have been eliminated.

KNOWN ISSUES

If the command line interface is used to disable the

web server, the encrypted web service (HTTPS) stops

operating.  They will be made independently selectable

in a future release.

When attempting to upgrade the firmware on a unit which

has password authentication enabled, the initial release

of the firmware would fail.  This current release

includes a workaround to this behavior by allowing the

user to disable passwords during the time period of the

firmware upgrade.

If it becomes necessary to DOWNGRADE the firmware from

version B (or greater) back to version A, it is necessary

to clear the persistent storage BEFORE downgrading because

the version A firmware has a defect in the way it handles

unexpected data in the persistent storage.  Failure to

clear the persistent storage during a DOWNGRADE to

A can result in an unusable unit.  The recommended

procedure, therefore, to preserve as many settings as

possible during a downgrade is to backup the configuration,

then clear the persistent storage, downgrade the firmware,

then restore the configuration.

In order to clear the persistent storage from the CLI

one can execute the "boot action=factory" command.  The

only web accessible method for clearing the storage is

available via the reset functionality in the

administrative pages at "admin/factory_defaults.htm".

When attempting to replace files in the file system, simply

overwrite the existing version of the file rather than deleting

the file first.  Attempting to delete the file first defeats

the internal file versioning maintained by the firmware, and

can confuse your browser's cache.

For the most consistent experience with the user interface,

it is suggested that you clear your Internet cache.

DOCUMENTATION ERRATA

Password authentication can be enabled in this release, but

authentication is OFF BY DEFAULT.  A security screen has been

added to the user interface to allow one to enable username

and password authentication.  When password checking is

enabled, the user will be challenged when accessing the

initial screen of the UI, and if the user uses a Digi Device

Discovery tool.

IP ASSIGNMENT NOTES

The Digi Connect ME now supports three IP assignment methods:

* Static IP address

* DHCP

* Auto-IP

If a static address is enabled, it will be used.

If a static address is not enabled, and DHCP is enabled, the

unit will use an address supplied by a DHCP server regardless

of the state of Auto-IP configuration.

If a static address is not enabled, and Auto-IP is enabled,

it will be used to generate an address ONLY if DHCP is

disabled, or if DHCP is enabled and a DHCP server has not

responded to the DHCP query.  If both are enabled, Auto-IP

has assigned an address, and then a DHCP server responds, the

Auto-IP address will be discarded and the DHCP address will

be used.

RESETTING THE UNIT

One feature introduced in the "C" revision of the Digi Connect

ME firmware is an enhanced ability for an embedded host system to

both soft reset the unit as well as reset the unit to its

factory defaults.

Both functions may be invoked via manipulation of pin 20

on the ME module's header:

* If the module is running (i.e. more than a few

seconds after power on), holding pin 20 low for

a second and then raising it will soft reset

the unit

* If pin 20 is held low for more than 10 seconds

from the power on or release from hard reset

of the unit, and then raised, it will reset

the unit to its factory default state

In either case, the action takes effect when the

pin is raised (reset released).

ADDITIONAL INFORMATION

The configuration save and restore tools will save every

configurable parameter (including IP configuration) except

for those related to password authentication.

ENABLING THE EMBEDDED WEB USER INTERFACE

The embedded web user interface is ALWAYS available at the

following URL:

http://ip-address-of-device/home.htm

It is also available as the default configuration interface

at the following URL IF an "index.htm" file is not already

present in your unit's filesystem:

http://ip-address-of-device

If your unit contains the Java Configuration Applet and associated

index.htm file in the file system, the Java Configuration Applet is

the default web user interface for the product.  If you would like to

use the embedded web user interface instead simply remove the index.htm

file as follows:

1. Access the administration web interface by entering the

following URL in a browser's URL window:

http://ip-address-of-device/admin/administration.htm

2. Select File Management under the Administration menu

3. Check the check box adjacent to the file, index.htm

4. Click the delete button

The Java Configuration Applet may remain in the unit's file system

without affecting the embedded web user interface.  If you would like

to use the Java Configuration Applet as an alternative, simply upload

the index.htm file as a file with an alternate name (e.g. applet.htm).

You would then be able to use the Java Configuration Applet by accessing

the following URL: http://ip-address-of-device/applet.htm

UPGRADING THE CONFIGURATION APPLET

Prior to upgrading the configuration applet, you may need to

remove the existing files.  The main reason for this is that the

earliest version of the applet used different filenames than

newer versions.  If you are simply uploading newer versions of

files with the same name, it is not necessary to first delete

the files.

Removing the existing files:

1. Access the administration web interface by entering the

following URL in in a browser's URL window:

http://ip-address-of-device/admin/administration.htm

2. Select File Management under the Administration menu.

3. Check the check box adjacent to the files you wish to

remove.

4. Click the delete button.

You must load the jar and HTML files onto the device (this

only has to be done once). When executed, the configuration

applet will connect back to the device it was loaded from.

Loading the updated configuration applet files:

1. Access the administration web interface by entering the

following URL in in a browser's URL window:

http://ip-address-of-device/admin/administration.htm

2. Select File Management under the Administration menu.

3. Click Browse. Locate and select the common.jar file

then click Open.

4. Click Upload.

5. Click Browse. Locate and select the configapp.jar file

then click Open.

6. Click Upload.

7. Click Browse. Locate and select the index.htm file

then click Open.

8. Click Upload.

9. Click Browse. Locate and select the config.ini file

then click Open.

10. Click Upload.

11. Click index.htm to run the configuration applet from

the device (to access the applet directly use the URL

http://ip-address-of-device/index.htm).

HISTORY

82000856B

"Reverse Telnet", i.e. making a telnet connection to a serial

port, has been added, including RFC 2217 support.

A command line interface has been added, accessible via telnet

to the standard telnet port (23).

The ability to autoconnect to a remote device via telnet has

been added.

The autoconnect feature has been extended to allow one to

autoconnect based on an arbitrary pattern string.

TCP socket connections now can be given a configurable

idle timeout for connection closure.

The "alarm" functionality has been extended to allow one to

generate email when pattern strings are present in the data

stream.

LPD server support has been added.

"Socket ID" support (the insertion of a static identifier

string into the first transmitted data packet for a TCP

socket session or into every datagram carrying data for a

serial port over UDP) has been added.

The serial over UDP functionality has been extended to

allow 64 destinations per serial port.

We have updated the Configuration Applet to support these

features.  You must upgrade the applet on your Digi Connect

ME in order to configure these new features from the UI.

See the section, UPGRADING THE CONFIGURATION APPLET, for

instructions.

CPU utilization reported by the web/applet interface is now

more representative of the average utilization of the system.

The serial over UDP functionality now correctly handles IP

broadcast addresses as valid UDP destinations.

The interface now automatically reboots after reverting to

factory defaults, as it previously implied it would.

82000856A

Initial Release.